| REVISION RECORD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DATE |  | APVD |  |  |  | DESCRIPTION |  |  |  |  |  |  |  |  |  |  | ECO |  |  | REV |  |  |  |  |  |  |  |  |
| 17.11.99 |  | YAEL H. |  |  |  | RELEASED |  |  |  |  |  |  |  |  |  |  | CJ31 |  |  | A |  |  |  |  |  |  |  |  |
| 56 | 55 |  | 54 | 53 |  |  | 52 | 51 |  |  | 50 | 49 |  | 48 |  | 47 |  | 46 |  |  | 45 | 44 |  |  |  | 43 | SHEET |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | REV |  |
| 42 | 41 |  | 40 | 39 |  |  | 38 | 37 |  |  | 36 | 35 |  | 34 |  | 33 |  | 32 |  |  | 31 | 30 |  |  |  | 29 | SHEET |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | REV |  |
| 28 | 27 |  | 26 | 25 |  |  | 24 | 23 |  |  | 22 | 21 |  | 20 |  | 19 |  | 18 |  |  | 17 | 16 |  |  |  | 15 | SHEET |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | REV |  |
| 14 | 13 |  | 12 | 11 |  |  | 10 | 9 |  |  | 8 | 7 |  | 6 |  | 5 |  | 4 |  |  | 3 | 2 |  |  |  | 1 | SHEET |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | A | A |  |  |  | A | REV |  |
| THIS DOCUMENT AND THE INFORMATION CONTAINED IN IT ARE PROPRIETARY AND CONFIDENTIAL TO MTI. NO PERSON IS ALLOWED TO COPY REPRINT REPRODUCE OR PUBLISH ANY PART OF THIS DOCUMENT NOR, DISCLOSE ITS CONTENTS TO OTHERS, NOR MAKE ANY USE OF IT NOR ALLOW OR ASSIST OTHERS TO MAKE ANY USE OF IT,- UNLESS BY THE PRIOR WRITTEN EXPRESS AUTHORIZATION OF MTI AND THEN ONLY TO THE EXTENT AUTHORIZED. (C) COPYRIGHT - MTI TECHNOLOGY AND ENGINEERING (1993) LTD. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MTI TECHNOLOGY AND ENGINEERING (1993) LTD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CONTRACT NO |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ANTENNA DIVISION |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DATE |  |  |  | SIGN |  |  |  |  |  | NAME |  |  | ACTION |
| TITLE |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 26.10.99 |  |  |  | Y.M |  |  |  |  |  | ### Y. MILLER |  |  | PREP |
| SPECIFICATION FOR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CHKD |
| MT-30044 ANTENNA |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 27.10.99 |  |  |  | M.H |  |  |  |  |  | ### M. HAIKIN |  |  | DSGN |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 27.10.99 |  |  |  | O.A |  |  |  |  |  | OFRA |  |  | PROJ |
| DWG NO |  |  |  |  |  |  |  |  | CLASSIFICATION |  |  |  | SIZE |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MFG |
| RD40721300C |  |  |  |  |  |  |  |  | NONE |  |  |  | A |  | 17.11.99 |  |  |  | MOTI |  |  |  |  |  | ### MOTOVICH |  |  | QA |
| SHEET 1 OF 3 |  |  |  |  |  |  |  |  |  | SCALE NONE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 961-7/96 |

1       Description :

This document specifies the electrical, mechanical and environmental characteristics  of

subscriber antenna MT-30044.

2.     Applicable Documents :

2.1 Outline Drawing :		RD40719600C – MT-30044 antenna outline drawing.

3.     General :

| MTI Part Number: | ## MT - 30044 |
| --- | --- |
| Type: | Subscriber, Directional |
| Radome: | Plastic, white |
| Base Plate: | Metal, Aluminum |
| # Regulatory Compliance: | ETSI EN 302 085 v1.1.1. (1999-01), TS1, TS2, TS3, TS4, TS5 |

4.    Electrical :

| # Frequency Range: | 3.4 GHz - 3.6 GHz |
| --- | --- |
| Gain: | 18 dBi (min) |
| 3 dB Beamwidth- (Azimuth) - (Elevation) | 18º (nom) 18º (nom) |
| # Polarization: | Linear (vertical or horizontal) |
| VSWR: | 1.5 : 1 (typ) 1.7:1 (max) |
| Power: | 1 watt |
| Input Impedance: | 50 ohms |

5.     Mechanical:

| # Input/RF Interface: | # N-TYPE female |
| --- | --- |
| # Dimensions: (LxWxH) | # 261 mm x 261 mm x 40 mm (max) |
| # Weight: | # 4 lbs (max) |
| # Mounting: | # 2" - 4" mounting bracket for pole mount. |
| # ICD: | # (to be supplied) |

6.     Environmental:

| # Temperature: (operating) | # - 40º C to + 65º C |
| --- | --- |
| # Lightning Protection: | # DC Grounded in accordance with IEC 1000 - 4 - 5 |
| # Wind: (survival) | # 220 Km/Hr |
| # Relative Humidity: | # 0% - 95%, in accordance with IEC 68 - 2 - 30. |
| # Ice Load: | # 25 mm radial |